SDN PROJECT REPORT: ARP & FIREWALL USING MININET + POX

1. Introduction

Software Defined Networking (SDN) is a modern networking paradigm in which the control plane is separated from the data plane. This allows centralized control of the network using a controller.

In this experiment, we use:

- Mininet to simulate the network topology

- POX controller to implement logic using OpenFlow rules

Objectives of the Experiment

- To understand communication between hosts in a network

- To study the working of ARP (Address Resolution Protocol)

- To implement a firewall rule using SDN

- To analyze packets using Wireshark

2. Understanding ARP

ARP (Address Resolution Protocol) is used to map an IP address to a MAC address.

Working of ARP

- A host knows the destination IP address

- It does not know the MAC address

- It sends an ARP Request (broadcast):

  - “Who has 10.0.0.X?”

- The destination host replies with an ARP Reply (unicast):

  - “I am 10.0.0.X”

This process happens before any ICMP (ping) communication.

3. Tools Used

- Mininet – Network simulation

- POX Controller – SDN controller

- OpenFlow – Communication protocol between switch and controller

- Wireshark – Packet capture and analysis

- Ubuntu VM – Execution environment

4. Network Topology

The topology consists of:

- 1 Switch (s1)

- 3 Hosts:

  - h1 → 10.0.0.1

  - h2 → 10.0.0.2

  - h3 → 10.0.0.3

All hosts are connected to a single switch.

5. Scenario 1: Normal Communication

In this scenario, no firewall rules are applied. All hosts are allowed to communicate.

Test 1: h1 → h2

- Packets transmitted: 3

- Packets received: 3

- Packet loss: 0%

Test 2: h2 → h3

- Communication successful

- Packet loss: 0%

Test 3: h1 → h3

- Communication successful

- Packet loss: 0%

6. Wireshark Analysis (Normal Case)

From the captured packets:

Observed Packets

- ARP Request → Broadcast

  - Example: Who has 10.0.0.3? Tell 10.0.0.2

- ARP Reply → Unicast

- ICMP Echo Request (Ping)

- ICMP Echo Reply

Key Observations

- ARP occurs before ICMP communication

- MAC address resolution is necessary for packet delivery

- After ARP, ICMP packets flow normally

Test 4: h1 → h2 (Blocked)

- Packets transmitted: 3

- Packets received: 0

- Packet loss: 100%

Test 6: h1 → h3 (Allowed)

- Communication successful

- Demonstrates selective filtering

8. Flow Table Analysis

The OpenFlow switch maintains flow entries:

Types of Rules

- Normal forwarding rules → Allow packets

- Firewall rule (high priority) → Drop packets

Important Observation

- Firewall rule has higher priority

- Matching packets are dropped at switch level

9. Wireshark Analysis (Firewall Case)

Observations

- ARP packets are still visible

- ICMP packets from h1 → h2 are absent

Conclusion

- ARP is not blocked

- ICMP packets are dropped by the firewall rule

10. Result Analysis

| Scenario | Communication | Packet Loss |
| --- | --- | --- |
| Normal | All hosts reachable | 0% |
| Firewall | h1 → h2 blocked | 100% |
| Firewall | Other communications OK | 0% |

Key Insights

- ARP always occurs before communication

- SDN controller dynamically installs flow rules

- Firewall rules override normal forwarding behavior

11. Conclusion

This experiment successfully demonstrates:

- The working of ARP in a network

- The role of an SDN controller (POX)

- Implementation of a firewall using OpenFlow rules

- Real-time packet inspection using Wireshark

The project highlights how SDN provides centralized control, flexibility, and programmability in managing network traffic.